Part 1: Configure a Router as a DHCP Server

R2>en

R2#conf t

R2(config)#ip dhcp excluded-

R2(config)#ip dhcp excluded-address

192.168.10.1 192.168.10.10

R2(config)#ip dhcp excluded-address

192.168.30.1 192.168.30.10

R2(config)#ip dhcp pool R1-LAN

R2(dhcp-config)#net

R2(dhcp-config)#network 192.168.10.0

255.255.255.0

R2(dhcp-config)#def

R2(dhcp-config)#default-router 192.168.10.1

R2(dhcp-config)#dns-

R2(dhcp-config)#dns-server 192.168.20.254

R2(dhcp-config)#exit

R2#conf t

R2(config)#ip dhcp pool R3-LAN

R2(dhcp-config)#net

R2(dhcp-config)#network 192.168.30.0

255.255.255.0

R2(dhcp-config)#def

R2(dhcp-config)#default-router 192.168.30.1

R2(dhcp-config)#dns

R2(dhcp-config)#dns-server 192.168.20.254

R2(dhcp-config)#exit

R2(config)#

Part 2: Configure DHCP Relay

Configure R1 as a DHCP relay agent:

R1>en

R1#conf t

R1(config)#int g0/0

R1(config-if)#ip help

R1(config-if)#ip helper-address 10.1.1.2

R1(config-if)#

Configure R3 as a DHCP relay agent:

R3>en

R3#conf t

R3(config)#int g0/0

R3(config-if)#ip help

R3(config-if)#ip helper-address 10.2.2.2

R3(config-if)#

Part 3: Configure R2 as a DHCP Client

R2>en

R2#conf t

R2(config)#int g0/1

R2(config-if)#ip address dhcp

R2(config-if)#no shutdown

R2#show ip interface brief

Part 4: Verify DHCP and Connectivity

Verify DHCP bindings:

R2#show ip dhcp binding

Verify configurations:

(For PC1)

C:\>ping 192.168.30.11

C:\>ping 192.168.10.1

C:\>ping 10.1.1.2

C:\>ping 192.168.30.1

C:\>ping 192.168.20.254

(For PC2)

C:\>ping 192.168.10.11

C:\>ping 192.168.30.1

C:\>ping 10.2.2.2

C:\>ping 192.168.10.1

C:\>ping 192.168.20.254